## Managed Workstations (Windows/macOS)

Data (what we must reliably collect)

- Real-time asset/session context: user, device type/OS, MAC/IP, switchport, LLDP/CDP, VLAN/VRF/segment. [digisoc.net]

- Posture signals at connect time: Forescout agent health; Intune/Jamf compliance; machine certificate + AD join; CrowdStrike sensor health/signature age; Qualys Cloud Agent installed & running. [digisoc.net], [blog.admindroid.com], [learn.jamf.com], [falconpy.io]

- Optional risk overlays: Defender Vulnerability Management (via Intune security tasks) and/or CrowdStrike Spotlight (scanless vuln posture). [blog.intunepckgr.com], [documentat...uronix.com]

Tooling / Systems (what must be in place)

- Forescout platform for discovery, posture, and enforcement (wired/wireless). [digisoc.net]

- Access switches capable of SNMP/REST control for per-port L3 ACL/dACL (default restricted L3; promote on pass). [digisoc.net]

- Cisco ISE (wireless): 802.1X auth + pxGrid/ANC to adjust roles/SGTs/dACLs on posture changes. [docs.forescout.com], [crowdstrik....github.io]

- MDM: Microsoft Intune and Jamf (with Jamf → Intune/Entra Device Compliance integration for macOS). [blog.admindroid.com], [learn.jamf.com]

- Endpoint security: CrowdStrike Falcon (and Spotlight if used); optional Microsoft Defender/MDVM integrated with Intune. [inventivehq.com], [documentat...uronix.com], [jeffreyappel.nl]

- SIEM/SOAR for unified telemetry and automation across NAC/ISE/EDR/MDM/Zscaler/NGFW. [digisoc.net]

Key Integrations (how the parts work together)

- Forescout ↔ Switch/WLC: SNMP/REST for ACL control; LLDP/CDP for accurate port-to-asset mapping. [digisoc.net]

- Forescout ↔ ISE (pxGrid): share posture; invoke ANC/SGT for wireless authorization changes. [docs.forescout.com], [crowdstrik....github.io]

- Forescout ↔ Intune/Jamf: consume compliance states to gate access; Jamf Device Compliance feeds Entra ID. [blog.admindroid.com], [learn.jamf.com]

- Forescout ↔ CrowdStrike/Spotlight (and/or Intune ↔ Defender/MDVM): import endpoint health/risk for post-admission downgrades/remediation. [falconpy.io], [documentat...uronix.com], [blog.intunepckgr.com]

Cultural & Technical Challenges (what can slow/derail execution)

- Legacy, mixed-vendor switching: uneven SNMP/ACL semantics; requires switch profiles and staged rollout. [digisoc.net]

- Wired without 802.1X: L3 ACLs are effective but less tamper-resistant—use DHCP snooping/IP-MAC binding and strict default restricted L3. [digisoc.net]

- MDM policy drift & exceptions: standardize compliance baselines; migrate Jamf Conditional Access to Device Compliance. [learn.jamf.com]

- Agent fatigue (EDR/NAC/Qualys): define gold-image order, Falcon sensor update policy rings, and auto-heal playbooks. [inventivehq.com]

- Role/rule parity across wired/wireless: keep Forescout outcomes aligned with ISE roles/SGTs to avoid inconsistent user experience. [docs.forescout.com]

- Telemetry overload: normalize NAC/ISE/EDR events in SIEM and correlate with Zscaler/ZPA/NGFW to reduce MTTR. [digisoc.net]

## IoT / OT (non-supplicant, poorly inventoried environments)

Data (what we must reliably collect)

- Agentless profiling: MAC/OUI, DHCP options, LLDP/CDP, passive protocol banners and traffic patterns to classify devices. [digisoc.net]

- Physical/logical context: switchport, VLAN/VRF, SGT; minimal allow-lists per device type and zone. [digisoc.net]

- Behavioral baselines: “known-good” flows to controllers/cloud; deviations trigger adaptive restriction/quarantine. [digisoc.net]

Tooling / Systems (what must be in place)

- Forescout profiling libraries/Device Cloud for higher classification confidence across IoT/OT verticals. [digisoc.net]

- Access infrastructure supporting MAB and per-port L3 ACL/dACL for default-deny enforcement (no 802.1X). [digisoc.net]

- Cisco ISE (pxGrid/ANC) for SGT-based segmentation and rapid containment of anomalous devices (WLAN/remote IoT). [docs.forescout.com]

- SIEM with OT-aware views to track inventory, behavior, and enforcement actions. [digisoc.net]

Key Integrations

- Forescout ↔ Access switches: push restricted L3 by default; promote to minimal allow-lists when profiling confidence is sufficient. [digisoc.net]

- Forescout ↔ ISE (pxGrid): apply ANC/SGT on classification changes or anomalous behavior; maintain plant/zone segmentation policy. [docs.forescout.com]

- Telemetry → SIEM/SOAR: export classification confidence, ACL/SGT changes, and anomalies for correlation with NGFW/ZT egress. [digisoc.net]

Cultural & Technical Challenges

- No system of record: NAC becomes de-facto inventory; institute discovery→owner attestation→allow-list workflow. [digisoc.net]

- Fragile/proprietary protocols: prefer passive discovery; validate ACL/SGT changes in vendor maintenance windows. [digisoc.net]

- Spoofing/shared ports: enforce port security, DHCP snooping, IP-MAC binding; anchor SGTs to ports where feasible. [digisoc.net]

- Change governance in plants: socialize “least-privilege by default,” keep break-glass roles with short TTL and full audit. [securitybo...levard.com]

- Signature drift/new device models: schedule profiling library updates; leverage Device Cloud feeds to sustain match accuracy over time. [digisoc.net]