# NAC, network admission policy enforcement. Discovery & Inventory Strategy (Updated for Switch-Based Enforcement Model, No RADIUS)

The Discovery & Inventory Strategy provides a unified, real-time method for detecting, profiling, and classifying every device on the wired LAN, forming the foundation of NAC enforcement, segmentation, posture validation, and operational safety. The strategy is built on a Forescout-centric, discovery-first architecture that uses switch telemetry—rather than RADIUS—combined with multi-signal fingerprinting and adaptive classification logic.

Forescout communicates directly with Cisco switches using SNMP and SSH/CLI to gather inventory data and enforce VLAN remapping, port blocking, and ACL-based restrictions without an authentication backend.

# 1. Discovery-First Architecture

The environment uses a network-centric discovery model that requires no agents or endpoint cooperation. Instead, Forescout relies on SSH-based CLI access and SNMP data collection across all switches.

### Switch Telemetry Collected

- VLAN tables

- MAC address tables

- ARP entries

- LLDP/CDP neighbor information

This method ensures complete visibility into all connected device types—managed laptops, IoT/OT assets, sensors, printers, VoIP phones, medical devices, and unknown endpoints.

# 2. Multi-Signal Device Fingerprinting

Forescout enriches raw switch telemetry using multiple identity signals:

- MAC OUI vendor lookup

- LLDP/CDP platform identifiers

- Switch-port attributes (incl. voice VLAN indicators)

- Historical VLAN placement and port migration

- Secure Connector posture data for managed devices

This approach is especially effective for agentless IoT/OT devices, which often cannot provide identity information themselves.

# 3. Weighted Classification Engine

A Python-based enrichment engine applies weighted scoring to the collected identity signals:

- +30 → LLDP/CDP platform tokens

- +25 → OUI manufacturer match

- +20 → Voice VLAN membership

- +10 → Historical VLAN alignment

- Posture override → For managed corporate endpoints

The resulting classification determines whether a device is Managed, IoT, OT, Telephony, Printer, or Unknown.

# 4. Normalized Device Record

All fingerprint and scoring data is consolidated into a normalized identity record containing:

- Device type

- MAC, IP, VLAN, port

- Posture indicators

- Assigned access tier (Prod, IoT/OT, Remediation, Quarantine)

This record is the authoritative source of truth for NAC policy decisions.

# 5. Forescout Enforcement via Cisco Switch Plugin

Forescout supports several different methods to support network enforcement. The Cisco L2/L3 switching infrastructure in place today, makes the using the Forescout Switch Plugin to enforce segmentation and containment actions the best approach. Supported actions include:

### VLAN Remapping

- Forescout applies “Assign to VLAN” or “Provision VLAN” actions via CLI.

### Port Blocking

- Using Switch Block, Forescout can disable a port (shutdown).

### ACL-Based Enforcement

- Apply or remove Access Port ACLs or Endpoint Address ACLs.

### Discovery-Driven State Updates

Device movement or posture changes trigger immediate switch reconfiguration.

# 6. Cisco Switch Configuration Requirements

To support discovery and enforcement, Cisco switches must be configured with:

### A. Local SSH User for Forescout Control and Device Profile discovery

Forescout uses only a CLI user to manipulate VLANs, ACLs, and port state.

username forescout privilege 15 secret <STRONG_PASSWORD>

username dev_discovery privilege 1 secret <STRONG_PASSWORD>

ip ssh version 2

line vty 0 4

login local

transport input ssh

Show more lines

### B. SNMP Access for Telemetry

This enables MAC/ARP table retrieval and port state monitoring.

snmp-server community FSNAC_RO ro

snmp-server community FSNAC_RW rw

snmp-server host <FORESCOUT_IP> version 2c FSNAC_RO

### C. LLDP/CDP for Fingerprinting

lldp run

cdp run

### D. NAC-Controlled Access Ports

Forescout may alter VLAN membership, block the port, or apply ACLs via SSH. These are the ports Forescout will dynamically modify:

interface GigabitEthernet1/0/12

description NAC_CONTROLLED_PORT

switchport mode access

switchport access vlan 10

spanning-tree portfast

Show more lines

### E. Optional: SNMP Traps for Rapid Detection

MAC move and port-up/down notifications improve detection speed.

snmp-server enable traps mac-notification change

mac address-table notification change

mac address-table notification interval 5

# 7. Continuous Inventory Maintenance

The Discovery Engine updates device records in real-time based on:

- MAC appearance/disappearance

- Port migration

- Posture changes

- Behavioral patterns

Forescout then immediately adjusts access tier or containment state using SSH/SNMP enforcement.

# 8. Strategy Outcomes

- Full device visibility across all wired-LAN endpoints

- Accurate IoT/OT discovery using multi-signal identity modeling

- Automated VLAN changes and blocking without RADIUS

- Reliable segmentation driven by normalized device identity

- Continuous real-time inventory updates